📡 Overview of WiFi CSI for Behaviour Recognition

- Passive sensing through WiFi CSI leverages signal distortions caused by human movement to recognize activities—no wearables or cameras needed.

- CSI vs RSS: CSI offers fine-grained channel data per subcarrier unlike RSS, which is unstable and averages signal changes.

- Deep learning focus: Authors propose using LSTM RNNs to replace traditional HMM and SVM classifiers, achieving higher recognition accuracy.

🔬 Key Techniques in Existing Literature

1. Signal Preprocessing

- Denoising: PCA filtering is emphasized to isolate reflections from mobile targets (discarding the first principal component).

- Phase Calibration: Raw CSI phase is corrupted by CFO and SFO—sanitization is needed for reliable use, though most systems avoid using phase data.

- Sampling Rate: 1 kHz frame rate is optimal—too low (e.g., 50 Hz) degrades recognition performance.

2. Feature Extraction

- STFT & DWT: Both used to translate CSI into frequency/time features. DWT is preferred due to its compactness and noise resilience.

- Energy Patterns: Movement speed is inferred by observing energy levels in specific frequency bands.

🤖 Machine Learning Models Evaluated

| Model | Pros | Cons |
| --- | --- | --- |
| Random Forest | Simple, interpretable | Weak at temporal sequence modeling |
| Hidden Markov | Captures sequence transitions | Requires feature engineering |
| LSTM RNN | Auto feature extraction, time memory | Longer training, but high accuracy |

- LSTM showed >75% accuracy across all activities with raw CSI input—no handcrafted features required.

🔍 Practical Considerations

- Environmental Variability: PCA helps reduce static influence. DWT and STFT provide speed-related features that generalize across environments.

- Phase Use Potential: While phase info is tricky due to noise, the paper suggests using phase differences between antennas might unlock spatial insights.

- Multi-User Challenges: Suggests MIMO-inspired separation techniques (like in WiSee) could enable multi-target activity differentiation.

🧠 Possible Directions for Your Research

Given your interest in transformers and TensorFlow optimization, here are a few angles worth exploring:

- Transformer-based Temporal Encoding: Instead of LSTM, use attention over CSI sequences for higher interpretability and better parallelization.

- End-to-End Pipeline with GPU Acceleration: Benchmark CSI-based recognition using TensorFlow with XLA and Apple Silicon profiling (fits your current focus).

- Phase-enhanced Embeddings: Combine amplitude and sanitized phase difference for richer input features—could benefit from multi-head attention.