Slide 1: Title Slide

• Project: Human ID & Activity Prediction using Wi-Fi Signals

• Leveraging Wi-Fi Channel State Information (CSI) and RSSI

• Your Name

• Date

--------------------------------------------------------------------------------

Slide 2: Project Objectives

• Original Dataset Aim: The primary aim of the paper is to present a resource for Wi-Fi-based Human Activity Recognition (HAR) in various indoor environments, including line-of-sight (LOS) and non-line-of-sight (NLOS) configurations.

• Our Project Objectives:

1. To detect the activity action type from Wi-Fi signals as input, aligning directly with the dataset's Human Activity Recognition (HAR) purpose.

2. To detect the ID of the person whose activity is sent in the form of Wi-Fi signals as input, leveraging the individual subject data available in the dataset.

--------------------------------------------------------------------------------

Slide 3: Dataset Overview - "Wi-Fi-based HAR Dataset"

• Source Paper Title: "A dataset for Wi-Fi-based human activity recognition in line-of-sight and non-line-of-sight indoor environments".

• Authors: Baha’ A Alsaify, Mahmoud M Almazari, Rami Alazrai, Mohammad I Daoud.

• Institutions: Department of Network Engineering and Security, Jordan University of Science and Technology, and Department of Computer Engineering, German Jordanian University.

• Publication Date: November 18, 2020.

• Purpose: To provide researchers with a dataset to test methodologies on both line-of-sight (LOS) and non-line-of-sight (NLOS) environments, encompassing various human movements, such as walking, falling, turning, and pen pick up from the ground.

--------------------------------------------------------------------------------

Slide 4: Key Features of the Dataset

• Participants:

◦ Data collected from 30 different healthy subjects (28 males, 2 females).

◦ Detailed information available for each subject: gender, age, weight, and height.

◦ Mean ± standard deviation age, height, and weight values were 22.7 ± 2.95 years, 178.37 ± 8.4 cm, and 81.9 ± 18.16 Kg, respectively.

• Experiments & Activities:

◦ Comprised of 5 distinct experiment classes (C1-C5).

◦ These experiments involve a total of 12 unique human activities (A1-A12).

▪ Examples include falling from sitting/standing positions, walking from transmitter to receiver, turning, sitting still, standing up, and picking up a pen from the ground.

◦ Each subject performed 20 trials for each of the 5 experiments, totaling 3000 recorded trials (30 subjects × 5 experiments × 20 trials).

• Environments:

◦ Data captured in 3 distinct indoor environments.

◦ 2 Line-of-Sight (LOS) configurations (a research laboratory and a university hallway).

◦ 1 Non-Line-of-Sight (NLOS) configuration (with an 8 cm thick wooden wall barrier).

• Data Types:

◦ Channel State Information (CSI) and Received Signal Strength Indicator (RSSI) values were recorded.

◦ 90 CSI values associated with each received packet (1 transmitting antenna, 3 receiving antennas, 30 subcarriers).

◦ RSSI over each of the three receiver antennas (RSSI_a, RSSI_b, RSSI_c).

• Data Format & Naming:

◦ Raw data stored as comma-separated values (.csv) files.

◦ Files are named systematically as "Ex_Sy_Cz_Ai_Tk.csv".

▪ 'S' indicates the Subject ID ({1, ..., 30}).

▪ 'C' indicates the Experiment Class ({1, ..., 5}).

▪ 'A' indicates the Activity ({1, ..., 12}).

▪ Example: "E1_S04_C3_A7_T17.csv" refers to data for Subject 4, Activity 7 (turning), Experiment Class 3 (walking between the transmitter and the receiver), in Environment 1, Trial 17.

--------------------------------------------------------------------------------

Slide 5: Feasibility of Human ID Prediction

• Direct Subject Identification: Each data file is explicitly linked to a unique Subject ID (S1 to S30) through its naming convention ("Ex_Sy_Cz_Ai_Tk.csv"), providing a direct label for training an identification model.

• Individualized Biometric Data: The dataset includes the gender, age, weight, and height for each of the 30 participants. These physical attributes can subtly influence how individuals interact with Wi-Fi signals, potentially creating unique "signatures".

• Sensitivity of Wi-Fi Signals:

◦ CSI and RSSI are highly sensitive to human presence and movement.

◦ It is plausible that even when performing the same 12 activities, different individuals will generate distinct Wi-Fi signal patterns due to variations in their body mechanics, size, and interaction with the environment.

• Ample Data for Training: The extensive collection of 20 trials for each of 5 experiment classes (encompassing 12 activities) per subject provides a rich dataset to train machine learning models to differentiate between the unique Wi-Fi signal characteristics of each individual.

• Beyond HAR: Although designed for HAR, the comprehensive, per-subject data collection enables broader research, including identifying individuals based on their unique Wi-Fi signal interactions.

--------------------------------------------------------------------------------

Slide 6: Use Case: Enhanced Security & Monitoring

• Addressing Limitations of Traditional Biometrics:

◦ Traditional biometrics (e.g., facial recognition, fingerprint scans) can be vulnerable to forgery (e.g., masks, fake fingerprints) [User Query].

◦ They often require active participation (e.g., looking at a camera, placing a finger) and are typically line-of-sight based.

• Unique Advantages of Wi-Fi-based Human ID & Activity Recognition:

◦ Passive and Non-Intrusive Monitoring: Our system leverages ambient Wi-Fi signals (CSI and RSSI values), which are passively collected, requiring no direct interaction from the individual.

◦ Robust Identification in Covert Scenarios: The project aims to detect the ID of a person using unique Wi-Fi signal interactions [Conversation History, Objective 2]. This could provide a secondary or primary layer of authentication that is difficult to spoof or bypass.

◦ Activity Anomaly Detection: Beyond ID, the ability to detect activity action types [Conversation History, Objective 1] allows for the identification of unusual or prohibited activities within a secured area (e.g., unauthorized movement patterns, falls).

◦ Non-Line-of-Sight (NLOS) Capability: A significant advantage is the system's ability to operate effectively in non-line-of-sight environments, even through an 8 cm thick wooden wall. This means monitoring can occur behind barriers or through walls, providing security where traditional cameras or optical sensors might fail.

• High-Security Zone Applications:

◦ Ideal for high-security zones like ISRO, NASA, or Parliament House [User Query], where access control and constant vigilance are paramount.

◦ Restricted Areas: Monitoring entry/exit and activities in highly restricted or classified areas without requiring visible sensors.

◦ Covert Surveillance: Detecting unauthorized presence or specific behaviors in areas where overt cameras are undesirable or easily avoided.

◦ Medical Alert Systems: The dataset's potential to distinguish a falling incident from other regular daily activities is highly valuable for real-time safety monitoring in sensitive or isolated environments.

--------------------------------------------------------------------------------

Slide 7: Data Collection Environment & Equipment

• Equipment:

◦ Two desktop computers, each with an Intel 5300 network interface card (NIC).

◦ One configured as a transmitter (1 antenna), the other as a receiver (3 antennas).

• Software:

◦ The CSI tool was utilized to capture and process Wi-Fi packets.

• Configuration:

◦ Operated at 2.4 GHz frequency band, channel 3, 20 MHz bandwidth.

◦ Sampling rate of 320 packets/second.

◦ Utilized a Multi-Input Multi-Output (MIMO) 1x3 Wi-Fi system (1 transmitting antenna, 3 receiving antennas).

• Experimental Setup:

◦ Subjects performed activities in an area between the transmitter and receiver.

◦ Distances between transmitter and receiver varied by environment (e.g., 3.7m in lab, 7.6m in hallway, 5.44m in NLOS with barrier).

◦ Pre-set timing diagrams with beep sounds were used to guide subjects through activities uniformly.

--------------------------------------------------------------------------------

Slide 8: Dataset for Wi-Fi-based Human Activity Recognition & Identification: Model Approaches

• Focus: Addressing "HAR Activity Action Type" and "ID of the Human" using the provided dataset.

• Source: A dataset for Wi-Fi-based human activity recognition in line-of-sight and non-line-of-sight indoor environments.

• Authors: Baha’ A Alsaify, Mahmoud M Almazari, Rami Alazrai, Mohammad I Daoud.

• Institution: Jordan University of Science and Technology.

--------------------------------------------------------------------------------

Slide 9: Our Two Target Variables

1. Human Activity Recognition (HAR) Activity Action Type

◦ Goal: Classify 12 unique human activities (A1-A12) across 5 distinct experiment classes (C1-C5).

◦ Examples of Activities: Falling from sitting position (A1, A2, A3), falling from standing position (A3, A4, A5), walking (A6, A7, A8, A9), sitting down and standing up (A1, A4, A10, A11), picking a pen from the ground (A12).

◦ Input Data: Primarily Channel State Information (CSI) and Received Signal Strength Indicator (RSSI) values.

2. ID of the Human

◦ Goal: Identify 1 of 30 distinct subjects (S1-S30) who performed the activities.

◦ Data Support: Each data file is explicitly named with a "Sy" (Subject ID), ranging from S1 to S30.

◦ Subject Details: The dataset includes gender, age, weight, and height for all 30 healthy subjects (28 males, 2 females). These individual attributes can influence Wi-Fi signal patterns.

◦ Input Data: Also uses CSI and RSSI values, which are sensitive to individual presence and movement characteristics.

--------------------------------------------------------------------------------

Slide 10: Dataset Overview for Both Objectives

• Comprehensive Dataset: Comprised of five experiments performed by 30 different subjects in three distinct indoor environments (line-of-sight and non-line-of-sight).

• Extensive Trials: 3000 recorded trials in total (30 subjects × 5 experiments × 20 trials).

• Key Data Inputs per Wi-Fi Packet:

◦ Channel State Information (CSI):

▪ 90 CSI values presented as complex numbers.

▪ Derived from 1 transmitting antenna, 3 receiving antennas, and 30 subcarriers.

◦ Received Signal Strength Indicator (RSSI):

▪ RSSI over each of the three receiver antennas (RSSI_a, RSSI_b, RSSI_c), measured in dB.

• Purpose: The dataset provides researchers with the ability to "test their developed methodologies" on various human movements and environmental conditions.

--------------------------------------------------------------------------------

Slide 11: Model Development Approaches: One vs. Two Models

• Core Question: Do we need to develop two separate models for "HAR Activity Action Type" and "ID of the Human," or can a single model predict both?

• Sources' Position: The provided sources describe the dataset's utility for researchers to "test their developed methodologies" and "assess the performance of Wi-Fi-based HAR systems". They do not prescribe a specific methodology or algorithm architecture.

• Feasibility: Given the structure and labeling of the dataset, both approaches are feasible for addressing your target variables.

--------------------------------------------------------------------------------

Slide 12: Approach 1: Developing Two Separate Models

• Model 1: HAR Activity Classifier

◦ Input: CSI and RSSI data from Wi-Fi packets.

◦ Output: Classifies the specific human activity (1 of 12 types).

◦ Optimization: Independently trained and optimized to achieve high accuracy in distinguishing between activities (e.g., identifying a "falling down" incident).

• Model 2: Human ID Classifier

◦ Input: Same CSI and RSSI data.

◦ Output: Identifies the specific subject (1 of 30 IDs).

◦ Optimization: Independently trained and optimized to differentiate between individuals based on their unique Wi-Fi signal patterns.

• Advantage: Allows for independent tuning and optimization for each specific prediction task, potentially maximizing accuracy for each target individually.

--------------------------------------------------------------------------------

Slide 13: Approach 2: Developing One Multi-Output Model

• Single Model Architecture:

◦ Takes shared CSI and RSSI data as its input.

◦ Features multiple output layers or "heads" to predict both target variables simultaneously.

• Outputs:

◦ Output Head 1: Predicts the HAR Activity Action Type (12 classes).

◦ Output Head 2: Predicts the ID of the Human (30 classes).

• Key Advantages:

◦ Leverages Shared Features: A single model can learn common, underlying features from the Wi-Fi signals that are relevant to both activity and individual identity.

◦ Potential for Efficiency: May be more computationally efficient to train and deploy one integrated model.

◦ Implicit Relationship Learning: The model could potentially learn subtle interactions between how different individuals perform various activities, as physical characteristics (age, height, weight) can influence movement patterns and thus Wi-Fi signal perturbation.

--------------------------------------------------------------------------------

Slide 14: Conclusion: Choice of Methodology

• The dataset provides rich, labeled Wi-Fi signal data (CSI, RSSI), along with explicit labels for 12 activity types and 30 subject IDs.

• This comprehensive data foundation makes it suitable for developing solutions for both target variables.

• The decision to implement two separate models or one multi-output model is a design choice that depends on the specific research goals, desired performance characteristics, and the complexity the developer wishes to manage. Both approaches offer valid pathways for utilizing this dataset to advance Wi-Fi-based human activity recognition and individual identification.