(PDF) Human Activity Recognition Through Augmented WiFi CSI Signals by Lightweight Attention-GRU

📚 Literature Review: Wi-Fi CSI-Based Human Sensing [CSI Dataset for Wireless Human Sensing on 80 MHz Wi-Fi Channels | IEEE DataPort]

1. Introduction

Wireless sensing has gained significant traction due to its ability to unobtrusively interpret human activities and identities using Wi-Fi signals. Among emerging methods, the use of Channel State Information (CSI) derived from commodity IEEE 802.11ac devices provides high-resolution signal data that can be exploited for human activity recognition, person identification, and people counting.

2. Dataset Foundation

The CSI dataset developed by Meneghello et al. leverages Wi-Fi channel fluctuations caused by human motion within multipath-rich environments. Key highlights include:

- Over 13 hours of CSI data across diverse indoor environments.

- 242 OFDM sub-channels and 4 monitor antennas, enhancing spatial resolution.

- Data acquired using Nexmon CSI tool on modified ASUS and Netgear routers operating on 80 MHz channels.

3. Experimental Design

A controlled communication link between two Wi-Fi routers ensures consistent packet transmission, with acquisition via a dedicated monitor router. The data flow includes:

- Configured routers using OpenWrt OS and hostapd for AP setup.

- Traffic generated via iPerf3 (~173 packets/sec), ensuring steady sampling intervals.

- CFR data stored as .mat files for further processing and structured analysis.

This setup maintains measurement integrity while promoting replication in future studies.

4. Structure of the Dataset

Organized into 26 subfolders, the dataset serves three primary sensing applications:

- Activity Recognition (AR): 18 folders including 8 activity types (walking, running, jumping, etc.).

- Person Identification (PI): Data organized by subject ID and movement trace.

- People Counting (PC): Traces captured for varying group sizes, enabling density estimation.

Each trace is formatted as a matrix of dimensions (N×Nant)×M(N \times N_{ant}) \times M, where Nant=4N_{ant} = 4 antennas and M=242M = 242 sub-channels.

5. Algorithmic Validation & Tools

The SHARP algorithm, introduced in this dataset’s companion research, validates the dataset’s applicability for robust, environment-agnostic activity recognition. Key contributions include:

- Phase sanitization to correct hardware-induced offsets.

- Sub-channel sign inversion on ranges [-63, 122] to mitigate artifacts.

- GitHub-hosted processing scripts and SHARP implementation support quick deployment in sensing pipelines.

6. Research Value and Applications

This dataset provides a reproducible benchmark for:

- Developing machine learning models for passive human sensing.

- Enhancing CSI-based feature extraction methods for dynamic indoor environments.

- Enabling cross-domain generalization by including seven environment types from bedrooms to university labs.

Its emphasis on standardized naming conventions, structured metadata, and open-source tooling makes it valuable for both academic exploration and industry prototyping.

7. Conclusion

Meneghello et al.’s work lays a strong foundation for advancing non-intrusive human sensing through Wi-Fi CSI data. By supporting multiple sensing modalities and offering robust preprocessing tools, this dataset catalyzes deeper investigation into remote activity recognition, behavioral analysis, and smart environment integration.